Detector – Specifications

Detector Type:	Cylindrical MWPC

Experiment:		MUCAP

Responsible:		P. Kammel, UIUC

- Diam. Anodes 				640 mm

- Diam. Cathodes outer				648 mm

- Angle of  cathode-Stripes inner		44.308° right screw

- Angle of  cathode-Stripes outer		45.738° left screw

- Diam. Cathodes inner				632 mm

- Diam. Shielding outer				652 mm

- Diam. Shielding inner				628 mm

- Diam. Stesalitring outer			662 mm

- Diam. Stesalitring inner			615 mm

- Width cathode-Stripes				4.444 period.

- Number of cathode-Stripes inner		320

- Number of cathode-Stripes outer		320

- Angle of  cathode-Stripes inner		44.308° right screw

- Angle of  cathode-Stripes outer		45.738° left screw

- Readout of cathode-Stripes			Downstream only

- Number of Anode Wires			1024

- Wire spacing					1.963 mm

- Diam. of Wires				20

- Material of Wires				Tungsten (Gold plated)

- Mech. Wire-Tension				0.45 - 0.6 N

- Active Length					800

- Halfgap					4 mm

- Length of Cylinders				860

- Total Length (w/o prints)			910

- Material of Cathode-Foils			50 Kapton /Al

- Material of Shielding-Foils			50 Kapton /Al