More Beam Studies

http://ten.npl.uiuc.edu/~kammel/doc_mulan/

Peter Kammel

1	Overview	2

2	Studies	2

2.1	jts and sensitivity to Q8/Q9	3

2.2	Differences between Jaap and Paul model, reality	7

2.2.1	Transport	7

2.2.2	Revmoc	10

2.3	Doublet-doublet, triplet-triplet, doublet-triplet optimization	17

3	Conclusions and further work	27

3.1	Conclusions	27

3.1.1	basic beamline geometry	27

3.1.2	Basic measuring program	27

3.2	Further work	29

4	Appendix: Summer 2001 tunes: Paul (x1) and Jaap(d1)	30

# Overview

This study is a follow-up to Jaap’s piE3 beam report (J1) and Peter’s discussion paper (P1) at the MuLan meeting. Several issues addressed in these reports are more systematically studied here, including

- kicker sensitivity to Q8/Q9

- differences between different models and reality

- specific studies for available beam geometry in MuLan 2002 run

- measurement program for MuLan 2002 run

Table 1: summary main tunes

# Studies

## jts and sensitivity to Q8/Q9

Changed distance Q8,Q9 from

5.00           'Q8  '      0.31400    -0.14759    12.90000;

3.0            '    '      0.61700;

5.00           'Q9  '      0.31400     0.17785    12.90000;

to

5.00           'Q8  '      0.31400    -0.26    12.90000;

3.0            '    '      0.21700;

5.00           'Q9  '      0.31400     0.271    12.90000;

Figure 1: jts tune

Figure 2: jt revmoc

Figure 3: jts revmoc

The Q8 setting determine the size of the x beam in the kicker region, the Q9 settings determine whether the beam is parallel or not. Thus both affect the divergence x’ in the kicker region.

First we study Q9 for 0.271 , 0.285 (+5%, jts9p) and 0.258 (-5% jts9m) kG. In tranport the effect at SLXY seem small, so we don’t even try to refocus the triplet. –5% gives small improvement, +5% gives deterioriation. Probably this effect can be fixed with triplet optimzation, as it is less evident in x’.

Figure 4: Q9 variation

Figure 5: Q8 field reduction

Figure 6: +- 5% variation of Q9 (first, second row),  much smaller Q8 (3rd row).

Next we study Q8. We make the beam a bit smaller and tune Q9 to mimimize x’ in the kick region (jts8m)

5.00           'Q8  '      0.31400    -0.100    12.90000;

3.0            '    '      0.21700;

5.00           'Q9  '      0.31400    +0.190    12.90000;

The separation seems a bit worse. A much smaller x envelope needs a polarity change in Q8/Q9. As Gerco is studying smaller kicker gaps, this study will not be done here.

## Differences between Jaap and Paul model, reality

| effect |  | comment |
| --- | --- | --- |
| 1 | element distances |  |
| 2 | target distance 1.5 m | required by MuLan |
| 3 | bend poleface curvature |  |
| 4 | quad PSI fringe field corrections |  |
| 5 | slits retract to 9.1 cm, separator 13 cm gap |  |
| 6 | quad pole faces r=12.0 cm |  |
| 7 | turn on separator |  |

### Transport

First correct 1 and 2, retune with achromatic and 1st order target focus condition (jtn.dat).

Figure 7: jtn.dat

Next study bend pole face curvature. Small effect.

Figure 8: with/without pole face curvature

Next PSI(GIOS) fringe field corrections for quads. Not such a big effect.

.

Figure 9: with/without fringe field correction, 1 st order also shown

Retune imposing achromaticity and first order target focus in SLXY and F (jtf.dat).

Figure 10: jtf.dat 1st and 2nd order, compared to jtn

Table 2: magnet settings for optimized tune with/without fringe corrections

Conclusion of this exercise within transport:

- updated geometry should be used (jtn.dat)

- pole face curvature small effect

- PSI fringe field corrections rather small local effect, but discrepancies propagate thorugh long channel. Retuning with fringe field corrections (jtf.dat) differs from jtn by up to 10%. Effect most significant in the last focusing triplet, but smaller retuning of achromaticity and SLXY focus required. Direct comparison of doublet/riplet with Zgoubi could clarify if PSI corrections more realistic. Otherwise experimental tuning of last 10% required.

### Revmoc

First we fix the geometries and pole curvatures a’la jtn.dat. We also include the pole curvatures and redo a 1st order tranport tune. The resultsin file is jtr.dat and translated into input5.jtr. We ignore the fringe fields as they are not included in revmoc. Then we include the other Monte Carlo specific corrections by comparing input5.jtr with Paul’s turte file (bend curvature not supported in revmoc).

Figure 11: jtr.dat, start for revmoc studies

/*PLOT* jtr/

0

1.000000       '    '      0.30000   150.00000     2.00000    30.00000

0.00000     4.00000     0.02800;

17.             '    ' ;

3.0            '    '      0.59750;

5.00           'Q1  '      0.47400     0.59112    12.00000;

3.0            '    '      0.12000;

5.00           'Q2  '      0.47400    -0.41200    12.00000;

3.0            '    '      0.62000;

5.00           'Q3  '      0.47400     0.18584    12.00000;

3.0            '    '      0.72130;

16.             'KR1E'     12.0        -0.9                ;

16.             'KR1A'     13.0         1.0                ;

16.00           '    '      5.00000     9.00000;

16.00           '    '      7.00000     0.30000;

16.00           '    '      8.00000     2.80000;

2.0            '    '     24.00000;

4.000          'B1  '      0.65950     1.48302     0.00000;

2.0            '    '     24.00000;

3.0            '    '      0.40330;

3.0            'SEX1'      0.32000;

3.0            'FS1 '      0.40350;

3.0            '    '      0.14500;

5.00           'Q4  '      0.39000     0.63094    12.50000;

3.0            '    '      0.21000;

5.00           'Q5  '      0.39000    -0.45737    12.50000;

3.0            'FS2 '      0.13300;

3.0            '    '      0.20000;

5.00           'Q6  '      0.39000    -0.27195    12.50000;

3.0            '    '      0.21000;

5.00           'Q7  '      0.39000     0.53321    12.50000;

3.0            '    '      0.82850;

3.0            'SEX2'      0.32000;

3.0            '    '      0.37630;

20.0            '    '    180.00000;

16.             'KR2E'     12.0        -0.4                ;

16.             'KR2A'     13.0         4.0                ;

16.00           '    '      5.00000     9.00000;

16.00           '    '      7.00000     0.340000;

16.00           '    '      8.00000     2.80000;

2.0            '    '     22.65000;

4.000          'B2  '      0.69420     1.40891     0.00000;

2.0            '    '     12.85400;

20.0            '    '    180.00000;

-10. 1.0 6.0 0.0 .0001 /B16/ ;

-10. 2.0 6.0 0.0 .0001 /B26/ ;

3.0            '    '      1.23620;

5.00           'Q8  '      0.31400    -0.26    12.90000;

3.0            '    '      0.21700;

5.00           'Q9  '      0.31400     0.271    12.90000;

3.0            '    '      0.64500;

3.0            'SEPA'      1.00000;

3.0            '    '      0.70000;

3.0            'KICK'      1.00000;

3.0            '    '      0.58600;

5.00           'Q10 '      0.31400    -0.50936    12.90000;

3.0            '    '      0.19600;

5.00           'Q11 '      0.31400     0.83509    12.90000;

3.0            '    '      0.19600;

5.00           'Q12 '      0.31400    -0.50936    12.90000;

3.0            'SLXY'      0.80000;

-10. 2. 1.  0.0  .0001 /W21/ ;

-10. 4. 3.  0.0  .0001 /W43/ ;

3.0            '    '      1.20000;

5.00           'Q13 '      0.36600     0.54713    12.70000;

3.0            '    '      0.13200;

5.00           'Q14 '      0.36600    -0.84757    12.70000;

3.0            '    '      0.13200;

5.00           'Q15 '      0.36600     0.54713    12.70000;

3.0            'F   '      1.50000;

-10. 2. 1.  0.0  .0001 /W21/ ;

-10. 4. 3.  0.0  .0001 /W43/ ;

-10. -3.0 4.0 0.0 .0001 /R34/ ;

-10. -1.0 2.0 0.0 .001 /R12/ ;

SENTINEL

SENTINEL

input5.jtr

1 jtr

T  T  .00001  0.00001  0.  0.  0.  0.00

D  BEG  0.00001  100.

D  0.59750  11.40

D  AQ1  0.00001 11.40

QUAD  Q1  0.47400    0.59112  12.00

D  0.00001  11.40

D  0.12000  11.40

QUAD  Q2  0.47400   -0.41200  12.00

D  0.00001  11.40

D  0.62000  11.40

QUAD  Q3  0.47400    0.18584  12.00

D  0.00001  11.40

D  0.72130  15.00  -15.00  8.50  -8.50

FRIN  9.00  0.3  2.8

ENTR  24.0

BEND  B1  0.65950  1.48302  0.  0.  0.028

EXIT  24.0

D  0.00001  15.00  -15.00  8.50  -8.50

D  0.40330  11.900

SEXT  SEX1  0.320000   0.00000  12.50

D  0.00001  11.900

D  0.40350  11.900

D  0.00001  9.10  -9.10  9.10  -9.10

D  0.14500  11.90

QUAD  Q4  0.39000    0.63094  12.50

D  0.00001  11.90

D  0.21000  11.90

QUAD  Q5  0.39000   -0.45737  12.50

D  0.00001  11.90

D  0.1330   11.90

D  0.00001  9.10  -9.10  9.10  -9.10

D  0.20000  11.90

QUAD  Q6  0.39000   -0.27195  12.50

D  0.00001  11.90

D  0.21000  11.90

QUAD  Q7  0.39000    0.53321  12.50

D  0.00001  11.90

D  0.82850  11.90

SEXT  SEX2  0.32000    0.00000  12.50

D  0.3763  15.00  -15.00  8.50  -8.50

ROTATE 180.

FRIN  9.00  0.34  2.8

ENTR  22.65

BEND  B2  0.69420  1.40891  0.  0.  0.028

EXIT  12.854

D  0.00001  15.00  -15.00  8.50  -8.50

ROTATE 180.

D  1.23620  12.30

QUAD  Q8  0.31400   -0.26  12.90

D  0.00001  12.30

D  0.21700  12.30

QUAD  Q9  0.31400    0.271  12.90

D  0.00001  12.30

D  0.64500  6.5  -6.5  10.0  -10.0

SEP  POSI  0.700  0.0  0.0  0.0  0.0  0.0  0.0  0.0

D  0.00001  6.5  -6.5  10.0  -10.0

D  1.00000  7.5  -7.5  10.0  -10.0

SEP  KICK  1.00  0.0  0.0  0.0  0.0  0.0  0.0  0.0

!SEP  KICK  1.00  1.5  0.0  0.0  0.0  0.0  0.0  0.0   ! kick on

D  0.00001  7.5  -7.5  10.0  -10.0

D  ANSP  0.58600  12.30

QUAD  Q10  0.31400   -0.50936  12.90

D  0.00001  12.30

D  0.19600  12.30

QUAD  Q11  0.31400    0.83509  12.90

D  0.00001  12.30

D  0.19600  12.30

QUAD  Q12  0.31400   -0.50936  12.90

D  0.00001  12.30

D  SLXY  0.80000  11.0  -11.0  15.  -15.

!D  SLXY  0.80000  1.0  -1.0  2.5  -2.5      ! collimator active

D  1.20000  12.1

QUAD  Q13  0.36600    0.54713  12.70

D  0.00001  12.10

D  0.13200  12.10

QUAD  Q14  0.36600   -0.84757  12.70

D  0.00001  12.10

D  0.13200  12.10

QUAD  Q15  0.36600    0.54713  12.70

D  AQ15  0.00001  12.10

D  F  1.50000  100.

XSIZE 0.6001   400.00001

YSIZE 4.0001  100.00001

P 0.02800  0.0042  0.028  .00001  0.   0.00

!P 0.02800  0.0000  0.028  .00001  0.   0.00     ! zero delta

MASS  *  .105659  0.  0.  1000000.

1.  .05

GROUP  1.  1000.000  0. 0.0  0.  1.

NSPAC 10. -1.

2.  2.  2.  2.  2.  2.  2.   2.  2.  2.

X  BEG   50.  1.

DX BEG   50.  400.

X  ANSP  50.  20.

DX ANSP  50.  30.

P  ANSP  50.  0.0042  0.028

DX ANSP  50.  60.

P  SLXY  50.  0.0042  0.028

X  SLXY  50.  4.

X  SLXY  50.  6.

Y  SLXY  50.  6.

X  SLXY  50.  6.

X  F     50.  10.

Y  SLXY  50.  6.

Y  F     50.  10.

X  AQ15  50.  30.

X  F     50.  10.

Y  AQ15  50.  30.

Y  F     50.  10.

DX ANSP  50.  60.

X  SLXY  50.  6.

END

Proschka, Paul’s turtle file for mixed mode

/PIE3 Run12, 27.4 MeVc, LB32, p34 ASKs set to bend by 60 degree/

100000

13. 10.0 ;   (observe all magnet apertures)

16. 3.0 206.77 /MU/ ;

16. 20.0 .0 ;  (homogeneous initial distribution)

1. .2 200.0 4.0 200.0 .0 10.0 0.0274 /BEAM/ ;

17. /2nd/ ;

6.  1. 0.15 /BEAX/ ; (slit in x)

6.  3. 2.   /BEAY/ ;  (slit in y)

16. 190. 0. 100. ; (store histo on metafile)

50. 11. 22. 38. 0.1 /HI01/ ;  (histo stuff 50xx)

50.1  1. -.4 .4  0.01 /ACX/ ;

50.1  3. -7. 7.  0.2 /ACX/ ;

50.1  2. -250. 250. 5. /ACXR/ ;

50.1  4. -80. 80. 2. /ACYR/ ;

3. .32 ;  (drift)

6. 1. 10. 3. 10. /TESH/ ; (define aperture due to shield near TgtE)

(half axis of ellipse)

3. .28 ;

16. 100.0 16.0 16.0 ;   (quads have circ aperture of r=16)

16. 101.0 11.5 ; (quads have hyperbolic apertures)

16. 25.0 .157 ; (quad fringe field sequence)

16. 26.0 -.023 ;

16. 27.0 .049 ;

16. 28.0 -.306 ;

2. 1000.00 ;

5. .474 .5487 12.0 /QTD1/ ;

2. 1000.00 ;

3. .1195 ;

2. 1000.00 ;

5. .474 -.3577 12.0 /QTD2/ ;

2. 1000.00 ;

3. .62 ;

2. 1000.00 ;

5. .474 .1169 12.0 /QTB1/ ;

2. 1000.00 ;

3. .6955 ;

16. 12.0 -.9 /KR1E/ ;  (bend in/out radii)

16. 13.0 1.0 /KR1A/ ;

16. 5.0 9.0 ;   (bend fringe)

16. 7.0 .3 /K1/ ;

16. 8.0 2.8 /K2/ ;

2. 24.0 /IN1/ ; (face rotation)

4. .7 1.367 .0 /ASK1/ ;

2. 24.0 /OUT1/ ;

3. .38 ;

18. .3 .2192 16.0 /HSA1/ ;

3. .4135 /FS71/ ;

6. 1. 9.1 /71OU/;  (slits)

6. 3. 9.1 /71LR/;

3. .1405 ;

16. 100.0 16.0 16.0 ;

16. 101.0 12.0 ; (different hyp boundary)

16. 25.0 .117 ; (diff fringe corrections)

16. 26.0 -.0113 ;

16. 27.0 .036 ;

16. 28.0 -.265 ;

2. 1000.00 ;

5. .39 .5482 12.5 /QSK1/ ;

2. 1000.00 ;

3. .21 ;

2. 1000.00 ;

5. .39 .1204 12.5 /QSK2/ ;

2. 1000.00 ;

3. .1666 /FS72/ ;

6. 1. 9.1 /72OU/; (slits)

6. 3. 9.1 /72LR/;

3. .1666 ;

2. 1000.00 ;

5. .39 -.5728 12.5 /QSK3/ ;

2. 1000.00 ;

3. .21 ;

2. 1000.00 ;

5. .39 .5689 12.5 /QSK4/ ;

2. 1000.00 ;

3. .8385 ;

18. .3 .0424 16.0 /HSA2/ ;

( 18. .3 .000 16.0 /HSA2/ ;)

3. .355 ;

20. 180.0 ;   (rotation)

16. 12.0 -.4 /KR1E/ ;

16. 13.0 4.0 /KR1A/ ;

16. 5.0 9.0 ;

16. 7.0 .34 /K1/ ;

16. 8.0 2.8 /K2/ ;

2. 22.65 /IN2/ ;

4. .7 1.367 .0 /ASK2/ ;

2. 12.854 /OUT2/ ;

20. 180.0 ;

3. .5475 ;

18. .3 0.0313 16.0 /HSD1/ ;

3. .36 ;

16. 100.0 16.0 16.0 ; (quad boundaries again)

16. 101.0 12.0 ;

2. 1000.00 ;

5. .314 .3601 12.9 /QSB1/ ;

2. 1000.00 ;

3. .217 ;

2. 1000.00 ;

5. .314 -.030 12.9 /QSB2/ ;

2. 1000.00 ;

3. 0.2 ;

(now the 320 ISO shutter and "balg")

(now the 320 ISO shutter and "balg" and Sep entrance)

3. 0.295 ;

(now rotate the beam by 90 degree to exchange x and y)

(the SEP instruction assume the E-field parallel to x)

(here, x is vertical and the E-field horizontal)

20. -90. ;   (rotation)

16. 3.0 206.77 /MU/ ;

-16. 3.0 1. /E+/ ;

16. 5. 11. /SDY/ ;  (width/2 ?)

16. 4. 6.5 /SDX/ ;   (gap/2 ?)

27. .01   .0  .0   /S1/ ;     (L m, E kV/cm, B kG)

27. .10   .0 -.005  /S2/ ;

27. .05   .0 -.0126  /S3/ ;

27. .05  4.55 -.04  /S4/ ;

27. .10  9.1 -.131  /S5/ ;

27. .10  9.1 -.142 /S6/ ;

27. .10  9.1 -.142  /S6/ ;

27. .10  9.1 -.131  /S7/ ;

27. .10  9.1 -.106  /S8/ ;

27. .10  9.1 -.0733  /S9/ ;

27. .05  4.55 -.04  /S10/;

27. .05  .0  -.0126  /S11/;

27. .10  .0  -.005  /S12/;

27. .0   .0   .0   /S13/;

16. 4. 60. ;

16. 5. 12. ;

20. 90. ;

(end of Sep, next ISO 320 shutter with additional tube)

3. 0.24 ;

3. 0.196 ;

2. 1000.00 ;

5. .314 -.6795 12.9 /QSB3/ ;

2. 1000.00 ;

3. .196 ;

2. 1000.00 ;

5. .314 .8904 12.9 /QSB4/ ;

2. 1000.00 ;

(from viewing the Quads in the area the QSB74 stops now)

(there is no additional drift length, so, "our" drift length start now)

3. 0.814 ;

53.1 ;

( 16. 180.0 77.0 1.0 ;)

3. 0. /TAR/;

SENTINEL

SENTINEL

Figure 12: jt, ignore F, because not optimized on 1.5 m

Figure 13: jtr with updates and refinements

Thus jtr pretty consistent with jt. Magnet and geometry changes (apart from Q8,Q9 and F) less than 5 %. Transmission about 10% smaller, probably because of 13 cm separator gap. Typical transmission table for events. Details are stored in tunes.xls spreadsheet.

Table 3: jtr transmission

Next we check the effect of the separator. We select 5KV/cm and 64G (ideally it should be 65.3G, but revmoc prefers slightly lower values (input5.jtp). There is a slight extension of the right tails, but the left tails stay practically the same.

SEP  POSI  0.700  5.0  0.064  0.0  0.0  0.0  0.0  0.0

Figure 14: jtp

Figure 15: more comparison jtr and jtp, small sep effects

Finally we check the effect of the SLXY position starting with jtr. We change

3.0            'SLXY'      0.80000;   to 	3.0            'SLXY'      1.20000

3.0            '    '      1.20000; 	   to	3.0            '    '      0.80000;

and reoptimize. The magnification at SLXY changes but the separation is similar to jtr.

Figure 16: jtx

/*PLOT* jtx/

0

1.000000       '    '      0.30000   150.00000     2.00000    30.00000

0.00000     4.00000     0.02800;

17.             '    ' ;

3.0            '    '      0.59750;

5.00           'Q1  '      0.47400     0.59112    12.00000;

3.0            '    '      0.12000;

5.00           'Q2  '      0.47400    -0.41200    12.00000;

3.0            '    '      0.62000;

5.00           'Q3  '      0.47400     0.18584    12.00000;

3.0            '    '      0.72130;

16.             'KR1E'     12.0        -0.9                ;

16.             'KR1A'     13.0         1.0                ;

16.00           '    '      5.00000     9.00000;

16.00           '    '      7.00000     0.30000;

16.00           '    '      8.00000     2.80000;

2.0            '    '     24.00000;

4.000          'B1  '      0.65950     1.48302     0.00000;

2.0            '    '     24.00000;

3.0            '    '      0.40330;

3.0            'SEX1'      0.32000;

3.0            'FS1 '      0.40350;

3.0            '    '      0.14500;

5.00           'Q4  '      0.39000     0.63094    12.50000;

3.0            '    '      0.21000;

5.00           'Q5  '      0.39000    -0.45737    12.50000;

3.0            'FS2 '      0.13300;

3.0            '    '      0.20000;

5.00           'Q6  '      0.39000    -0.27195    12.50000;

3.0            '    '      0.21000;

5.00           'Q7  '      0.39000     0.53321    12.50000;

3.0            '    '      0.82850;

3.0            'SEX2'      0.32000;

3.0            '    '      0.37630;

20.0            '    '    180.00000;

16.             'KR2E'     12.0        -0.4                ;

16.             'KR2A'     13.0         4.0                ;

16.00           '    '      5.00000     9.00000;

16.00           '    '      7.00000     0.340000;

16.00           '    '      8.00000     2.80000;

2.0            '    '     22.65000;

4.000          'B2  '      0.69420     1.40891     0.00000;

2.0            '    '     12.85400;

20.0            '    '    180.00000;

-10. 1.0 6.0 0.0 .0001 /B16/ ;

-10. 2.0 6.0 0.0 .0001 /B26/ ;

3.0            '    '      1.23620;

5.00           'Q8  '      0.31400    -0.26    12.90000;

3.0            '    '      0.21700;

5.00           'Q9  '      0.31400     0.271    12.90000;

3.0            '    '      0.64500;

3.0            'SEPA'      1.00000;

3.0            '    '      0.70000;

3.0            'KICK'      1.00000;

3.0            '    '      0.58600;

5.00           'Q10 '      0.31400    -0.45479    12.90000;

3.0            '    '      0.19600;

5.00           'Q11 '      0.31400     0.75955    12.90000;

3.0            '    '      0.19600;

5.00           'Q12 '      0.31400    -0.45479    12.90000;

3.0            'SLXY'      1.20000;

-10. 2. 1.  0.0  .0001 /W21/ ;

-10. 4. 3.  0.0  .0001 /W43/ ;

3.0            '    '      0.80000;

5.00           'Q13 '      0.36600     0.60585    12.70000;

3.0            '    '      0.13200;

5.00           'Q14 '      0.36600    -0.90349    12.70000;

3.0            '    '      0.13200;

5.00           'Q15 '      0.36600     0.60585    12.70000;

3.0            'F   '      1.50000;

-10. 2. 1.  0.0  .0001 /W21/ ;

-10. 4. 3.  0.0  .0001 /W43/ ;

-10. -3.0 4.0 0.0 .0001 /R34/ ;

-10. -1.0 2.0 0.0 .001 /R12/ ;

SENTINEL

SENTINEL

## Doublet-doublet, triplet-triplet, doublet-triplet optimization

Based on the studies of the previous section we take jtp or jtr as the input file for optimization of the last two focussing combinations (for the transport studies jtr, ignoring the Wien filter is used).

As a guidance we first check jtr (jtrt indicates modifications of SLXY, AQ15 slits). The shaping of the y focus is mainly achieved with a SLXY slit in y. The AQ15 slit is only marginally used to cut the x tail. The F vs. AQ15/SLXY correlation plots indicate that smaller foci can be achieved with these slits at a cost of intensity. Comparison with jtp gives very close results, so – given the simplistic separator simulation – we stay with jtr.

The detailed studies below lead to the following conclusions

- jdr2 and jdr3 roughly equivalent, but 50 % loss compared to jtr.

- With same transmission jtrt allow much better y focus.

- jtx is similar to jtr.

Table 4: x and y width and transmission at F position

Figure 17: jtrt

Figure 18: jdr1, jdr2, jdr3

| tune | last quads |  |
| --- | --- | --- |
| jdr1 | 5.00 'Q10 ' 0.31400 0.42017 12.90000; 3.0 ' ' 0.19600; 5.00 'Q11 ' 0.31400 -0.41208 12.90000; 3.0 'SLXY' 1.20000; 3.0 ' ' 0.80000; 5.00 'Q12 ' 0.36600 -0.18 12.70000; 3.0 ' ' 0.13200; 5.00 'Q13 ' 0.36600 0.435 12.70000; 3.0 ' ' 0.13200; 5.00 'Q14 ' 0.36600 0.0 12.70000; 3.0 'F ' 1.50000; | no reasonable y focus in revmoc |
| jdr2 | 5.00 'Q10 ' 0.31400 -0.54930 12.90000; 3.0 ' ' 0.19600; 5.00 'Q11 ' 0.31400 +0.6837 12.90000; 3.0 'SLXY' 1.20000; 3.0 ' ' 0.80000; 5.00 'Q12 ' 0.36600 +0.5994 12.70000; 3.0 ' ' 0.13200; 5.00 'Q13 ' 0.36600 -0.8741 12.70000; 3.0 ' ' 0.13200; 5.00 'Q14 ' 0.36600 +0.5994 12.70000; 3.0 'F ' 1.50000; | looks promising. The x focus at SLXY is very narrow (HWZM=1.2cm). However, ~50% intensity is lost compared to jtr. Advantage compared to jdr3 that SLXY relatively narrow in both planes (measurable with MWPC). |
| jdr3 | 5.00 'Q10 ' 0.31400 0.39 12.90000; 3.0 ' ' 0.19600; 5.00 'Q11 ' 0.31400 -0.3235 12.90000; 3.0 'SLXY' 1.20000; 3.0 ' ' 0.80000; 5.0 'Q12 ' 0.36600 -0.4001 12.70000; 3.0 ' ' 0.13200; 5.00 'Q13 ' 0.36600 0.6388 12.70000; 3.0 'F ' 1.50000; | 2 doublet combination also promising. Similar performance as jdr3, but again at least 50% loss. |

Table 5: jd comparison

Figure 19:jdr2

Figure 20: jdr3, jtr narrow,  jtx

A tight selection in x and y, as imposed for kciking and to make the y focus narrow (jtrt narrow), changes the momentum acceptance compared to the open slit situation in jtr. This is due to the chromatic aberations in second order.

Figure 21: comparison p acceptance for wide and narrow SLXY cuts

1 jdr3

T  T  .00001  0.00001  0.  0.  0.  0.00

D  BEG  0.00001  100.

D  0.59750  11.40

D  AQ1  0.00001 11.40

QUAD  Q1  0.47400    0.59112  12.00

D  0.00001  11.40

D  0.12000  11.40

QUAD  Q2  0.47400   -0.41200  12.00

D  0.00001  11.40

D  0.62000  11.40

QUAD  Q3  0.47400    0.18584  12.00

D  0.00001  11.40

D  0.72130  15.00  -15.00  8.50  -8.50

FRIN  9.00  0.3  2.8

ENTR  24.0

BEND  B1  0.65950  1.48302  0.  0.  0.028

EXIT  24.0

D  0.00001  15.00  -15.00  8.50  -8.50

D  0.40330  11.900

SEXT  SEX1  0.320000   0.00000  12.50

D  0.00001  11.900

D  0.40350  11.900

D  0.00001  9.10  -9.10  9.10  -9.10

D  0.14500  11.90

QUAD  Q4  0.39000    0.63094  12.50

D  0.00001  11.90

D  0.21000  11.90

QUAD  Q5  0.39000   -0.45737  12.50

D  0.00001  11.90

D  0.1330   11.90

D  0.00001  9.10  -9.10  9.10  -9.10

D  0.20000  11.90

QUAD  Q6  0.39000   -0.27195  12.50

D  0.00001  11.90

D  0.21000  11.90

QUAD  Q7  0.39000    0.53321  12.50

D  0.00001  11.90

D  0.82850  11.90

SEXT  SEX2  0.32000    0.00000  12.50

D  0.3763  15.00  -15.00  8.50  -8.50

ROTATE 180.

FRIN  9.00  0.34  2.8

ENTR  22.65

BEND  B2  0.69420  1.40891  0.  0.  0.028

EXIT  12.854

D  0.00001  15.00  -15.00  8.50  -8.50

ROTATE 180.

D  1.23620  12.30

QUAD  Q8  0.31400   -0.26  12.90

D  0.00001  12.30

D  0.21700  12.30

QUAD  Q9  0.31400    0.271  12.90

D  0.00001  12.30

D  0.64500  6.5  -6.5  10.0  -10.0

SEP  POSI  0.700  5.0  0.064  0.0  0.0  0.0  0.0  0.0

D  0.00001  6.5  -6.5  10.0  -10.0

D  1.00000  7.5  -7.5  10.0  -10.0

SEP  KICK  1.00  0.0  0.0  0.0  0.0  0.0  0.0  0.0

D  0.00001  7.5  -7.5  10.0  -10.0

D  ANSP  0.58600  12.30

QUAD  Q10  0.31400   0.39    12.90

D  0.00001  12.30

D  0.19600  12.30

QUAD  Q11  0.31400   -0.3235     12.90

D  SLXY  1.20000  11.0  -11.0  15.  -15.

D  0.80000  12.1

QUAD  Q12  0.36600     -0.4001  12.70

D  0.00001  12.10

D  0.13200  12.10

QUAD  Q13  0.36600    0.6388  12.70

D  AQ15  0.00001 6. -6.   15. -15.

D  0.00001  12.10

D  F  1.50000  100.

XSIZE 0.6001   400.00001

YSIZE 4.0001  100.00001

P 0.02800  0.0042  0.028  .00001  0.   0.00

MASS  *  .105659  0.  0.  1000000.

1.  .05

GROUP  1.  1000.000  0. 0.0  0.  1.

NSPAC 10. -1.

2.  2.  2.  2.  2.  2.  2.   2.  2.  2.

X  F     50.  10.

Y  F     50.  10.

X  ANSP  50.  30.

DX  ANSP 50.  60.

P  ANSP  50.  0.0042  0.028

DX ANSP  50.  60.

P  SLXY  50.  0.0042  0.028

X  SLXY  50.  10.

X  SLXY  50.  10.

Y  SLXY  50.  30.

X  SLXY  50.  10.

X  F     50.  10.

Y  SLXY  50.  30.

Y  F     50.  10.

X  AQ15  50.  30.

X  F     50.  10.

Y  AQ15  50.  30.

Y  F     50.  10.

DX ANSP  50.  60.

X  SLXY  50.  10.

END

1 jtx

T  T  .00001  0.00001  0.  0.  0.  0.00

D  BEG  0.00001  100.

D  0.59750  11.40

D  AQ1  0.00001 11.40

QUAD  Q1  0.47400    0.59112  12.00

D  0.00001  11.40

D  0.12000  11.40

QUAD  Q2  0.47400   -0.41200  12.00

D  0.00001  11.40

D  0.62000  11.40

QUAD  Q3  0.47400    0.18584  12.00

D  0.00001  11.40

D  0.72130  15.00  -15.00  8.50  -8.50

FRIN  9.00  0.3  2.8

ENTR  24.0

BEND  B1  0.65950  1.48302  0.  0.  0.028

EXIT  24.0

D  0.00001  15.00  -15.00  8.50  -8.50

D  0.40330  11.900

SEXT  SEX1  0.320000   0.00000  12.50

D  0.00001  11.900

D  0.40350  11.900

D  0.00001  9.10  -9.10  9.10  -9.10

D  0.14500  11.90

QUAD  Q4  0.39000    0.63094  12.50

D  0.00001  11.90

D  0.21000  11.90

QUAD  Q5  0.39000   -0.45737  12.50

D  0.00001  11.90

D  0.1330   11.90

D  0.00001  9.10  -9.10  9.10  -9.10

D  0.20000  11.90

QUAD  Q6  0.39000   -0.27195  12.50

D  0.00001  11.90

D  0.21000  11.90

QUAD  Q7  0.39000    0.53321  12.50

D  0.00001  11.90

D  0.82850  11.90

SEXT  SEX2  0.32000    0.00000  12.50

D  0.3763  15.00  -15.00  8.50  -8.50

ROTATE 180.

FRIN  9.00  0.34  2.8

ENTR  22.65

BEND  B2  0.69420  1.40891  0.  0.  0.028

EXIT  12.854

D  0.00001  15.00  -15.00  8.50  -8.50

ROTATE 180.

D  1.23620  12.30

QUAD  Q8  0.31400   -0.26  12.90

D  0.00001  12.30

D  0.21700  12.30

QUAD  Q9  0.31400    0.271  12.90

D  0.00001  12.30

D  0.64500  6.5  -6.5  10.0  -10.0

SEP  POSI  0.700  5.0  0.064  0.0  0.0  0.0  0.0  0.0

D  0.00001  6.5  -6.5  10.0  -10.0

D  1.00000  7.5  -7.5  10.0  -10.0

SEP  KICK  1.00  0.0  0.0  0.0  0.0  0.0  0.0  0.0

!SEP  KICK  1.00  1.5  0.0  0.0  0.0  0.0  0.0  0.0   ! kick on

D  0.00001  7.5  -7.5  10.0  -10.0

D  ANSP  0.58600  12.30

QUAD  Q10  0.31400   -0.45479  12.90

D  0.00001  12.30

D  0.19600  12.30

QUAD  Q11  0.31400    0.75955  12.90

D  0.00001  12.30

D  0.19600  12.30

QUAD  Q12  0.31400   -0.45479  12.90

D  0.00001  12.30

D  SLXY  1.20000  11.0  -11.0  15.  -15.

D  0.80000  12.1

QUAD  Q13  0.36600     0.60585  12.70

D  0.00001  12.10

D  0.13200  12.10

QUAD  Q14  0.36600    -0.90349   12.70

D  0.00001  12.10

D  0.13200  12.10

QUAD  Q15  0.36600     0.60585  12.70

D  AQ15  0.00001 6. -6.   4. -4.

D  F  1.50000  100.

XSIZE 0.6001   400.00001

YSIZE 4.0001  100.00001

P 0.02800  0.0042  0.028  .00001  0.   0.00

MASS  *  .105659  0.  0.  1000000.

1.  .05

GROUP  1.  100.000  0. 0.0  0.  1.

NSPAC 10. -1.

2.  2.  2.  2.  2.  2.  2.   2.  2.  2.

X  F     50.  10.

Y  F     50.  10.

X  ANSP  50.  20.

DX ANSP  50.  30.

P  ANSP  50.  0.0042  0.028

DX ANSP  50.  60.

P  SLXY  50.  0.0042  0.028

X  SLXY  50.  4.

X  SLXY  50.  6.

Y  SLXY  50.  6.

X  SLXY  50.  6.

X  F     50.  10.

Y  SLXY  50.  6.

Y  F     50.  10.

X  AQ15  50.  30.

X  F     50.  10.

Y  AQ15  50.  30.

Y  F     50.  10.

DX ANSP  50.  60.

X  SLXY  50.  6.

END

# Conclusions and further work

## Conclusions

### basic beamline geometry

- Present Q6, Q7 distance ok

- 1 m distance between SEP and KICK assumed, should be as short as possible

- KICK dimensions in present study: deflector electrodes 1 m long, gap 15cm, width 20 cm
Two beam pipe collimator inserts of this dimensions should be prepared in Boston (additional alternative insert geometry depends on Gerco’s studies).

- If possible the two last focusing elements should be two triplets for the summer run. This gives us the highest rate and flexibility. Second best choice is doublet, then triplet.

- SLXY should be located at 1.2 m from last Q, with distance 0.8 m to next Q.

- If possible we should have an AQ15 slit.

- Beam pipe should extend all the way to F for beam tests and have the full 30 cm diameter window (0.126 mm thick?). We don’t want to mix up beam properties and scattering effects.

### Basic measuring program

- geometries

| JP | Johny chamber close to window, SC behind last chamber |
| --- | --- |
| JM | Johny chamber close to window, SC behind first chamber |
| ES | telescope beam scan |
| RS | range scan |
| SiS | Silicon beam scan |
| BC | little beam chamber |

comments:

JP, JM pretty obvious

ES could be a larger area eye scintillator arrangement for muons, but RS probably more precise. Also eyes don’t have an electron veto. Perhaps just to a telescope with 5x5 cm2 scintillators.

RS is small scintillator telescope mounted on the beam scanner (see Figure 22: sketch of RS). It consists of

  - RS1 (0.1mm thick):  to determine low energy tails

  - RS2 (3mm thick): main mu stop counter

  - RS3(1cm thick): electron veto

High rate compatible. Potentially a collimator should be used to better define the acceptance. With that device we should be able to measure chromaticity and space vs p correlations. In particular we should be sensitive to low energy tails. This method should be compared to SiS measurement.

SiS alternative to RS or combination?

BC perhaps useful at F location.

Best detector arrangement should be discussed in more detail.

- measurements at SLXY

|  | measurements | particles |
| --- | --- | --- |
| 1 | JP phase space for promising tunes | pi- |
| 2 | JP augment with ES for large beam sizes |  |
| 3S | JM separator tunes | mu+ |
| 3 | JM for promising tunes |  |
| 4 | RS for best tunes |  |
| 3’ | as 3 but kicked beam |  |
| 4’ ? | as 4 ES or RS but kicked beam, see mu and e |  |

- measurements at F

|  | measurements | particles |
| --- | --- | --- |
| 3 | JM or BC | mu+ |
| 4 | RS for best tunes |  |
| 3’ | as 3 but kicked beam |  |
| 4’ | as 4 but kicked beam, see mu and e, PID |  |

mu- measurements in last week.

Figure 22: sketch of RS

## Further work

- Comparison with turtle (Gerco)
Originally we planned a comparison with turtle for one specific example. As a first comparison indicated some differences it would be worthwhile to redo the full optimization above for the best case to check whether one arrives at similar conclusion as with revmoc.

- Momentum selection studies (Gerco)
Mixed tune studies are certainly important to find a tune that allows momentum selection. Tentatively I found a such a tune, starting with the mixed setup for the first 3 quads and then tuning Q4-Q7 for achromaticity. 
In the jtr, jdr exercises above I improved the focus by sacrificing more than 50% of the beam. The y optimization was done primarily to reduce chromatic effects. Probably the same can be achieved by momentum selection, with the additional benefit of a smaller range width. A full optimization has to be worked out.

- Find better (i.e. higher  transmission) doublet-doublet or triplet-doublet combinations

- Kicker with smaller gap (Gerco). Scaling laws for RF supply.

- y kick studies (Peter?)
If time permits I will try that.

- sexupole corrections?
There are pretty strange behaviours in the x,x’ vs p plot. Can that we improved with sexupoles? Can the y focusing be improved with sexupoles?

- Zgoubi ?
Jaap recommends the use of Zgoubi, but also warns that a lot of work is involved. We would have to use the quad maps to properly account for end shimming. Who will do that job?

- Measuring program and tuning strategy (Peter&Paul)
We should have a plan 10 days in advance of the run. If hardware preparations are involved (beamline, detectors) even earlier.

- Contact Lee
We have to contact Lee concerning the pseudo kicker collimators.

- Prepare detectors

# Appendix: Summer 2001 tunes: Paul (x1) and Jaap(d1)

d1.dat

'*PLOT* d1'

(use Doornbos labels)

(no quad fringe field in original file-leave option in)

(sextupoles set to zero in original file)

0

1. .20 150.0 2.0 30.0 0.0 5.0 .0280 /BEAM/ ;

-17. /ZWEI/;

3. .5975 ;

-16. 25.0 .157 ;

-16. 26.0 -.023 ;

-16. 27.0 .049 ;

-16. 28.0 -.306 ;

-2. 1000.00 ;

(Q1 = QTD71)

5. .474 .60112 12.0 /Q1/ ;

-2. 1000.00 ;

3. .12 ;

-2. 1000.00 ;

(Q2 = QTD72)

5. .474 -.4120 12.0 /Q2/ ;

-2. 1000.00 ;

3. .62 ;

-2. 1000.00 ;

(Q3 = QTB71)

5. .474 .18584 12.0 /Q3/ ;

-2. 1000.00 ;

3. .72130 ;

(no pole face curvatures in original file)

-16. 12.0 -.9 /KR1E/ ;

-16. 13.0 1.0 /KR1A/ ;

16. 5.0 9.0 ;

16. 7.0 0.3   /K1/ ;

16. 8.0 2.8   /K2/ ;

2. 24.0       /IN1/ ;

(B1 = ASK71)

4. .65950 1.48302 .0 /B1/ ;

2. 24.0       /OUT1/ ;

3. .40130 ;

18. .320 0.000 16.0 /SEX1/ ;

3. .4135 /FS71/ ;

(fit for focus in both planes at first slits)

-10. -1.0 2.0 0.0 .001 /A12 / ;

-10. -3.0 4.0 0.0 .001 /A34 / ;

3. .1350 ;

-16. 25.0 .117 ;

-16. 26.0 -.0113 ;

-16. 27.0 .036 ;

-16. 28.0 -.265 ;

-2. 1000.00 ;

(Q4 = QSK71)

5. .39 .63212 12.5 /Q4/ ;

-2. 1000.00 ;

3. .21 ;

-2. 1000.00 ;

(note change of polarity from standard tunes)

(Q5 = QSK72)

5. .39 -0.45737 12.5 /Q5/ ;

-2. 1000.00 ;

3. .14150 /FS71/ ;

(fit for focus in non bend plane at second slits)

-10. -3.0 4.0 0.0 0.001 ;

3. .14150  ;

-2. 1000.00 ;

(Q6 = QSK73)

5. .39 -.27195 12.5 /Q6/ ;

-2. 1000.00 ;

(next drift should be 0.21)

3. .26 ;

-2. 1000.00 ;

(Q7 = QSK74)

5. .39 .52178 12.5 /Q7/ ;

-2. 1000.00 ;

3. .8285 ;

18. .320 0.000 16.0 /SEX2/ ;

3. .3763 ;

20. 180.0 ;

(no pole face curvature in original file)

-16. 12.0 -.4 /KR2E/ ;

-16. 13.0 4.0 /KR2A/ ;

16. 5.0 9.0 ;

16. 7.0 .34 /K1/ ;

16. 8.0 2.8 /K2/ ;

2. 22.65 /IN2/ ;

(B2 = ASK72)

4. .69420 1.40891 .0 /B2/ ;

2. 12.854  /OUT2/ ;

20. 180.0 ;

(fit for zero dispersion and ang dispersion)

-10. 1.0 6.0 0.0 .0001 /B16/ ;

-10. 2.0 6.0 0.0 .0001 /B26/ ;

3. .5475 ;

18. .320 0.00 16.0 /SEX3/ ;

3. .36 ;

-2. 1000.00 ;

(Q8 = QSB71)

5. .314 .43737 12.9 /Q8/ ;

-2. 1000.00 ;

(next drift is larger by 40 cm than present beam line)

3. .617 ;

-2. 1000.00 ;

(Q9 = QSB72)

5. .314 -.24501 12.9 /Q9/ ;

-2. 1000.00 ;

3. .64500 ;

-16. 3.0 206.77 /MU/ ;

3. 0.35 /SEP1/ ;

(fit for focus in both planes at center of separator)

-10. 2. 1. 0.0 0.001 /C12 /;

-10. 4. 3. 0.0 0.001 /C34 /;

3. 0.35 /SEP2/ ;

3. 0.5869  ;

-2. 1000.00 ;

(Q10 = QSB73)

5. .314 -.50942 12.9 /Q10/ ;

-2. 1000.00 ;

3. 0.1960 ;

-2. 1000.00 ;

(Q11 = QSB74)

5. .314 .83167 12.9 /Q11/ ;

-2. 1000.00 ;

(two long drifts before triplet)

3. 0.80 /SCOL/ ;

(fit for focus in both planes)

-10.  2.0  1.0  0.0  0.001 ;

-10.  4.0  3.0  0.0  0.001 ;

3.0  1.20 ;

(quad triplet before for last focus)

5.0  0.36600     0.69743    12.70000   /Q12 /;

3.0  0.13200 ;

5.0  0.36600    -0.92859    12.70000   /Q13 /;

3.0  0.13200;

5.0  0.36600     0.55342    12.70000   /Q14 /;

3.0  0.75000 /FIN /;

(fit for focus in both planes)

-10. 2.0  1.0   0.0   0.001;

-10. 4.0  3.0   0.0   0.001;

SENTINEL

SENTINEL

x1.dat

/*PLOT* x1/

0

1. .15 160.0 3.0 20.0 .0 0.0 .0287 /BEAM/ ;

-17. /ZWEI/;

3. .5975 ;

16. 25.0 .157 ;

16. 26.0 -.023 ;

16. 27.0 .049 ;

-16. 28.0 -.306 ;

2. 1000.00 ;

5. .474 .5694 12.0 /QTD1/ ;

2. 1000.00 ;

3. .12 ;

2. 1000.00 ;

5. .474 -.3709 12.0 /QTD2/ ;

2. 1000.00 ;

3. .62 ;

2. 1000.00 ;

5. .474 .1209 12.0 /QTB1/ ;

2. 1000.00 ;

3. .7213 ;

16. 12.0 -.9 /KR1E/ ;

16. 13.0 1.0 /KR1A/ ;

16. 5.0 9.0 ;

16. 7.0 .3 /K1/ ;

16. 8.0 2.8 /K2/ ;

2. 24.0 /IN1/ ;

4. .6595 1.5277 .0 /ASK1/ ;

2. 24.0 /OUT1/ ;

3. .4033 ;

18. .320 0.2263 16.0 /HSA1/ ;

3. .4135 /FS71/ ;

-10. -1.0 1.0 1.5 .001 /A11/ ;

-10. -1.0 2.0 0.0 .001 /A12/ ;

3. .1405 ;

16. 25.0 .117 ;

16. 26.0 -.0113 ;

16. 27.0 .036 ;

-16. 28.0 -.265 ;

2. 1000.00 ;

5. .39 .5684 12.5 /QSK1/ ;

2. 1000.00 ;

3. .21 ;

2. 1000.00 ;

5. .39 .1244 12.5 /QSK2/ ;

2. 1000.00 ;

3. .1666 /FS72/ ;

3. .1666 ;

2. 1000.00 ;

5. .39 -.5952 12.5 /QSK3/ ;

2. 1000.00 ;

3. .21 ;

2. 1000.00 ;

5. .39 .59 12.5 /QSK4/ ;

2. 1000.00 ;

3. .8285 ;

18. .320 0.0435 16.0 /HSD2/ ;

3. .3763 ;

20. 180.0 ;

16. 12.0 -.4 /KR2E/ ;

16. 13.0 4.0 /KR2A/ ;

16. 5.0 9.0 ;

16. 7.0 .34 /K1/ ;

16. 8.0 2.8 /K2/ ;

2. 22.65 /IN2/ ;

4. .6942 1.4228 .0 /ASK2/ ;

2. 12.854 /OUT2/ ;

20. 180.0 ;

-10. 1.0 6.0 0.0 .0001 /B16/ ;

-10. 2.0 6.0 0.0 .0001 /B26/ ;

3. .5662 ;

18. .320 0.0320 16.0 /HSD1/ ;

3. .350 ;

(no changes beyond this entry)

2. 1000.00 ;

5. .314 .3725 12.9 /QSB1/ ;

2. 1000.00 ;

3. .217 ;

2. 1000.00 ;

5. .314 -.031 12.9 /QSB2/ ;

2. 1000.00 ;

3. 0.2 ;

(now the 320 ISO shutter and "balg")

3. 0.27 ;

(now the SEP, first a drift until plates)

3. 0.175 ;

16. 3.0 206.77 /MU/ ;

(next line is drift up to center of Sep)

3. 0.35 /SEP/ ;

3. 0.35 ;

( 20. -90. ;)

( 21. 0.7 0.1 /SEP/ ;)

( 20. 90. ;)

(another drift until the end of SEP)

3. 0.175 ;

(next ISO 320 shutter with additional tube)

3. 0.215 ;

3. 0.196 ;

2. 1000. ;

5. .314 -.6795 12.9 /QSB3/ ;

2. 1000. ;

3. .196 ;

2. 1000. ;

5. .314 .8904 12.9 /QSB4/ ;

2. 1000. ;

(from viewing the Quads in the area the QSB74 stops now)

(there is no additional drift length, so, "our" drift length start now)

3. .814 ;

3. 0. /TAR/ ;

-10. 3.0 3.0 2.5 .001 /Y/ ;

-10. -3.0 4.0 0.0 .0001 /R34/ ;

-10. -1.0 2.0 0.0 .001 /R12/ ;

-10. 1.0 1.0 0.2 .001 /X/ ;

-10. -1.0 6.0 0.0 .001 /R16/ ;

13. 4.0 ;

SENTINEL

SENTINEL

transport distances from Paul’s evaluation