Jaap’s report

discussion and some studies

http://ten.npl.uiuc.edu/~kammel/doc_mulan/

Peter Kammel

1	Transport studies	2

2	Revmoc studies	11

3	Peter’s tunes and variants	22

3.1	jt1.dat: simple improved jt tune	22

3.2	pnb.dat: new nb kick tune	23

3.3	pd.dat: increased transmission (?) jd tune	26

4	Conclusion and tuning strategy	30

# Transport studies

Assumptions

- initial conditions

| HWHM | x (cm) | x’ (mrad) | y (cm) | y’ (mrad) | p |
| --- | --- | --- | --- | --- | --- |
| standard | 0.3 | 150 | 2 | 30 | 4 |
| test | 0.5 |  | 3 |  | 0 |

- layout

  - standard two symmetric triplets

  - optional two doublets

- curvature of bends neglected, marginal

- no PSI fringe field corrections for quads

Strategy and Performance

- beam properties at end of kicker, second order waist (i.e. dependent on p assumption)

| HWHM | x | x’ | y | y’ |
| --- | --- | --- | --- | --- |
| 1st order | 6.1 | 7.8 | 7.7 | 26.1 |
| 2nd order | 16.5 | 10.4 | 19.3 | 52.5 |

- chromatic second order corrections important, geometric aberrations small

- sextupoles turned off, introduce large non-chromatic aberrations

- beam properties at SLXY

| HWHM | x | x’ | y | y’ |
| --- | --- | --- | --- | --- |
| 1st order | 0.56 | 80.4 | 0.96 | 62 |
| 2nd order | 3.0 | 151 | 3.01 | 67 |

- beam properties at F

| HWHM | x | x’ | y | y’ |
| --- | --- | --- | --- | --- |
| 1st order | 0.54 | 84 | 0.84 | 70 |
| 2nd order | 3.72 | 171 | 5.44 | 85 |

Comments

- chromatic 2nd  order dominant, optimization in 2nd order necessary

- SLXY and F not perfect waist

- non bend plane needs study

- SLXY focus condition: x(SLXY)=R12 x’(KICK), R11=0
in 1st order independent of R12 !

Figure 1: jt.dat, first and second order

Figure 2: sin/cos, first order

Figure 3: p HWHM=4 %, 0%

Figure 4: jd.dat, first and second order

Figure 5: sin/cos

Figure 6: 4/0 % HWHM p

Kick

SLXY

F

Figure 7: jt tune,  selected T2jk

Figure 8: jt tune, selected T4jk

/*PLOT* jd/

0

1.000000       '    '      0.30000   150.00000     3.00000    30.00000

0.00000     4.00000     0.02800;

17.             '    ' ;

3.0            '    '      0.59750;

5.00           'Q1  '      0.47400     0.59112    12.00000;

3.0            '    '      0.11950;

5.00           'Q2  '      0.47400    -0.43200    12.00000;

3.0            '    '      0.62000;

5.00           'Q3  '      0.47400     0.18584    12.00000;

3.0            '    '      0.72130;

16.00           '    '      5.00000     9.00000;

16.00           '    '      7.00000     0.30000;

16.00           '    '      8.00000     2.80000;

2.0            '    '     24.00000;

4.000          'B1  '      0.65950     1.48302     0.00000;

2.0            '    '     24.00000;

3.0            '    '      0.40130;

3.0            'SEX1'      0.32000;

3.0            '    '      0.41350;

3.0            '    '      0.13500;

5.00           'Q4  '      0.39000     0.63212    12.50000;

3.0            '    '      0.21000;

5.00           'Q5  '      0.39000    -0.45737    12.50000;

3.0            '    '      0.14150;

3.0            '    '      0.14150;

5.00           'Q6  '      0.39000    -0.27195    12.50000;

3.0            '    '      0.26000;

5.00           'Q7  '      0.39000     0.52178    12.50000;

3.0            '    '      0.82850;

3.0            'SEX2'      0.32000;

3.0            '    '      0.37630;

20.0            '    '    180.00000;

2.0            '    '     22.65000;

4.000          'B2  '      0.69420     1.40891     0.00000;

2.0            '    '     12.85400;

20.0            '    '    180.00000;

3.0            '    '      1.23620;

5.00           'Q8  '      0.31400    -0.21759    12.90000;

3.0            '    '      0.61700;

5.00           'Q9  '      0.31400     0.20785    12.90000;

3.0            '    '      0.64500;

3.0            'SEPA'      1.00000;

3.0            '    '      0.70000;

3.0            'KICK'      1.00000;

3.0            '    '      0.58600;

5.00           'Q10 '      0.31400     0.41471    12.90000;

3.0            '    '      0.19600;

5.00           'Q11 '      0.31400    -0.33011    12.90000;

3.0            '    '      0.80000;

3.0            '    '      1.20000;

5.00           'Q12 '      0.36600    -0.43926    12.70000;

3.0            '    '      0.13200;

5.00           'Q13 '      0.36600     0.69583    12.70000;

3.0            'F   '      1.00000;

SENTINEL

SENTINEL

/*PLOT* jt/

0

1.000000       '    '      0.30000   150.00000     2.00000    30.00000

0.00000     4.00000     0.02800;

17.             '    ' ;

3.0            '    '      0.59750;

5.00           'Q1  '      0.47400     0.59112    12.00000;

3.0            '    '      0.11950;

5.00           'Q2  '      0.47400    -0.41200    12.00000;

3.0            '    '      0.62000;

5.00           'Q3  '      0.47400     0.18584    12.00000;

3.0            '    '      0.72130;

16.00           '    '      5.00000     9.00000;

16.00           '    '      7.00000     0.30000;

16.00           '    '      8.00000     2.80000;

2.0            '    '     24.00000;

4.000          'B1  '      0.65950     1.48302     0.00000;

2.0            '    '     24.00000;

3.0            '    '      0.40130;

3.0            'SEX1'      0.32000;

3.0            '    '      0.41350;

3.0            '    '      0.13500;

5.00           'Q4  '      0.39000     0.63212    12.50000;

3.0            '    '      0.21000;

5.00           'Q5  '      0.39000    -0.45737    12.50000;

3.0            '    '      0.14150;

3.0            '    '      0.14150;

5.00           'Q6  '      0.39000    -0.27195    12.50000;

3.0            '    '      0.26000;

5.00           'Q7  '      0.39000     0.52178    12.50000;

3.0            '    '      0.82850;

3.0            'SEX2'      0.32000;

3.0            '    '      0.37630;

20.0            '    '    180.00000;

2.0            '    '     22.65000;

4.000          'B2  '      0.69420     1.40891     0.00000;

2.0            '    '     12.85400;

20.0            '    '    180.00000;

3.0            '    '      1.23620;

5.00           'Q8  '      0.31400    -0.14759    12.90000;

3.0            '    '      0.61700;

5.00           'Q9  '      0.31400     0.17785    12.90000;

3.0            '    '      0.64500;

3.0            'SEPA'      1.00000;

3.0            '    '      0.70000;

3.0            'KICK'      1.00000;

3.0            '    '      0.58600;

5.00           'Q10 '      0.31400    -0.49776    12.90000;

3.0            '    '      0.19600;

5.00           'Q11 '      0.31400     0.82379    12.90000;

3.0            '    '      0.19600;

5.00           'Q12 '      0.31400    -0.49776    12.90000;

3.0            'SLXY'      0.80000;

3.0            '    '      1.20000;

5.00           'Q13 '      0.36600     0.61452    12.70000;

3.0            '    '      0.13200;

5.00           'Q14 '      0.36600    -0.91114    12.70000;

3.0            '    '      0.13200;

5.00           'Q15 '      0.36600     0.61452    12.70000;

3.0            'F   '      1.00000;

SENTINEL

SENTINEL

# Revmoc studies

(PK changed revmoc to include cernlib plotting)

Assumptions

- initial conditions: x(FWHM)=0.6, y(FWHM)=4 cm, p(FWHM)=15%

- specific elements

|  | x (cm) | y(cm) | desciption |
| --- | --- | --- | --- |
| KICK | 15 | 20 | kicker |
| ANSP |  |  | thin drift space after KICK |
| SLXY |  |  | kicker slit |
| AQ15 |  |  | collimator after last quad |
| F |  |  | target 1m after last quad |

Performance

- transmission , all slits open

|  | trans(%) | rate(6 cm) MHz | rate(4 cm) MHz |
| --- | --- | --- | --- |
| Q9 | 35.5 | 45 | 32 |
| F | 23.7 |  | 21.4 |

- momentum acceptance 9% FWHM, 10.5 % FWQM

- separation not dependent on x width source distribution (0.6 to 1 cm), but transmission decreases by 15%

- unkicked beam

  - SLXY x<3cm, y<6cm, slit of  1cm in x contains 90%

  - for of center momenta x’ mostly positive due to T2662, but in  fact small (fig.8)?

  - complixated x vs p plot at SLXY, tail to positive x side

- kicked beam

  - 1.5 kV/cm gives 21 mr (compared to divergence of 10 mr)

  - positive (up) kicking favourable

| kick | transmission( 1cm x SLXY) % | final |
| --- | --- | --- |
| 0 | 90 |  |
| -1.5 kV/cm | 1.3 |  |
| + 1.5 kV/cm | 0 |  |

- final beam spot

  - polarity of final triplet

  - collimator AQ15

|  | bend plane (dim in cm) |  |  | non-bend plane (dim in cm) |  |  | source | rate | trans |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| case | FWHM | FWQM | FWTM | FWHM | FWQM | FWTM | (cm) | (MHz) | (%) |
| trip bend foc | 1.8 | 2.4 | 3.3 | 3.0 | 4.5 | 6.6 | 4 | 20 | 24 |
| with slit,coll | 1.8 | 2.4 | 3.3 | 3.3 | 4.5 | 5.1 | 6 | 20 | 17 |
| trip nbend foc | 2.1 | 3.0 | 4.8 | 3.0 | 4.5 | 6.0 | 4 | 20 | 24 |
| with slit, coll | 1.8 | 2.4 | 3.0 | 3.3 | 4.5 | 5.1 | 6 | 20 | 17 |
| 10 (dublet) | 1.2 | 1.8 | 2.1 | 3.6 | 4.2 | 4.8 | 6 | 20 | 17 |
| 11 (dublet) |  |  |  |  |  |  | 4 |  | too small |

Figure 9: unkicked

Figure 10: unkicked

Figure 11: unkicked, SLXY to 1cm

Figure 12: unkicked, SLXY to 1cm, target beam spot

Figure 13: kicked with +1.5 kV/cm

1 doornew_bp_3a_report.rev april 16 2002

T  T  .00001  0.00001  0.  0.  0.  0.00

D  BEG  0.00001  100.

D  0.59750  11.40

QUAD  Q1  0.47400    0.59112  12.00

D  0.00001  11.40

D  0.11950  11.40

QUAD  Q2  0.47400   -0.42200  12.00

D  0.00001  11.40

D  0.62000  11.40

QUAD  Q3  0.47400    0.18584  12.00

D  0.00001  11.40

D  0.72130  15.00  -15.00  8.50  -8.50

FRIN  9.00  0.3  2.8

ENTR  24.0

BEND  B1  0.65950  1.48302  0.  0.  0.028

EXIT  24.0

D  0.00001  15.00  -15.00  8.50  -8.50

D  0.40130  11.900

SEXT  SEX1  0.320000   0.00000  12.50

D  0.00001  11.900

D  0.41350  99.90

D  0.13500  11.90

QUAD  Q4  0.39000    0.63212  12.50

D  0.00001  11.90

D  0.21000  11.90

QUAD  Q5  0.39000   -0.45737  12.50

D  0.00001  11.90

D  0.14150  99.90

D  0.14150  11.90

QUAD  Q6  0.39000   -0.27195  12.50

D  0.00001  11.90

D  0.26000  11.90

QUAD  Q7  0.39000    0.52178  12.50

D  0.00001  11.90

D  0.82850  11.90

SEXT  SEX2  0.32000    0.00000  12.50

D  0.3763  15.00  -15.00  8.50  -8.50

ROTATE 180.

ENTR  22.65

BEND  B2  0.69420  1.40891  0.  0.  0.028

EXIT  12.854

D  0.00001  15.00  -15.00  8.50  -8.50

ROTATE 180.

D  1.23620  12.30

QUAD  Q8  0.31400   -0.14759  12.90

D  0.00001  12.30

D  0.61700  12.30

QUAD  Q9  0.31400    0.17785  12.90

D  0.00001  12.30

D  0.64500  7.5  -7.5  10.0  -10.0

SEP  POSI  1.00  0.0  0.0  0.0  0.0  0.0  0.0  0.0

D  0.00001  7.5  -7.5  10.0  -10.0

D  0.64500  7.5  -7.5  10.0  -10.0

SEP  KICK  1.00  0.  0.0  0.0  0.0  0.0  0.0  0.0

D  0.00001  7.5  -7.5  10.0  -10.0

D  ANSP  0.58600  12.30

QUAD  Q10  0.31400   -0.49776  12.90

D  0.00001  12.30

D  0.19600  12.30

QUAD  Q11  0.31400    0.82379  12.90

D  0.00001  12.30

D  0.19600  12.30

QUAD  Q12  0.31400   -0.49776  12.90

D  0.00001  12.30

D  SLXY  0.80000  11.0  -11.0  15.  -15.

D  1.20000  12.1

QUAD  Q13  0.36600    0.61452  12.70

D  0.00001  12.10

D  0.13200  12.10

QUAD  Q14  0.36600   -0.91114  12.70

D  0.00001  12.10

D  0.13200  12.10

QUAD  Q15  0.36600    0.61452  12.70

D  AQ15  0.00001  12.10

D  F  1.00000  100.

XSIZE 0.6001   400.00001

YSIZE 4.0001  100.00001

P 0.02800  0.0042  0.028  .00001  0.   0.00

MASS  *  .105659  0.  0.  1000000.

1.  .05

GROUP  1.  100.000  0. 0.0  0.  1.

NSPAC 10. -1.

2.  2.  2.  2.  2.  2.  2.   2.  2.  2.

X  BEG  20.  1.

DX BEG  20.  400.

X  ANSP  20.  20.

DX ANSP  30.  30.

P  ANSP  30.  0.0042  0.028

DX ANSP  30.  60.

P  SLXY  30.  0.0042  0.028

X  SLXY  40.  4.

X  SLXY  30.  6.

Y  SLXY  30.  6.

X  SLXY  30.  6.

X  F   30.  9.

Y  SLXY  30.  6.

Y  F   30.  9.

X  AQ15  30.  30.

X  F   30.  9.

Y  AQ15  30.  30.

Y  F   30.  9.

DX ANSP  30.  60.

X  SLXY  30.  6.

END

PARTICLES FINALLY REJECTED FROM THOSE INITIALLY ACCEPTED

Z(M)   LAB   TYP  ENER    TOO LOW    MULT. SCAT. OUT     ABSORBED   NUC. SCAT. LOSS  3-BODY DECAYS   TOT # REJECT.   % LEFT

9      .000 T      TARG

9      .000 BEG    DRIFT

9      .598        DRIFT        .000           6173.000           .000            .000            .000        6173.000     	93.82700

9     1.072 Q1     QUAD

9     1.072        DRIFT        .000          16641.000           .000            .000            .000       16641.000     	77.18600

9     1.191        DRIFT        .000           3697.000           .000            .000            .000        3697.000     	73.48901

9     1.665 Q2     QUAD

9     1.665        DRIFT        .000           8403.000           .000            .000            .000        8403.000     	65.08601

9     2.285        DRIFT        .000            127.000           .000            .000            .000         129.000    	 64.95701

9     2.759 Q3     QUAD

9     2.759        DRIFT        .000              7.000           .000            .000            .000           7.000     		64.95000

9     3.480        DRIFT        .000           5863.000           .000            .000            .000        5863.000     	59.08701

9     3.480        FRING

9     3.480        ENTR

9     4.140 B1     BEND

9     4.140        EXIT

9     4.140        DRIFT

9     4.541        DRIFT

9     4.861 SEX1   SEXT

9     4.861        DRIFT        .000           1386.000           .000            .000            .000        1386.000     	57.70100

9     5.275        DRIFT

9     5.410        DRIFT        .000           7934.000           .000            .000            .000        7934.000     	49.76701

9     5.800 Q4     QUAD

9     5.800        DRIFT        .000             62.000           .000            .000            .000          63.000    		 49.70401

9     6.010        DRIFT        .000            479.000           .000            .000            .000         479.000    	 49.22501

9     6.400 Q5     QUAD

9     6.400        DRIFT        .000           1333.000           .000            .000            .000        1333.000     	47.89201

9     6.541        DRIFT

9     6.683        DRIFT

9     7.073 Q6     QUAD

9     7.073        DRIFT        .000            259.000           .000            .000            .000         259.000     	47.63301

9     7.333        DRIFT        .000           4051.000           .000            .000            .000        4051.000    	 43.58201

9     7.723 Q7     QUAD

9     7.723        DRIFT        .000           1830.000           .000            .000            .000        1830.000     	41.75201

9     8.551        DRIFT        .000               .000           .000            .000            .000           1.000     		41.75101

9     8.871 SEX2   SEXT

9     9.247        DRIFT        .000           1431.000           .000            .000            .000        1431.000     	40.32001

9     9.247        ROTAT

9     9.247        ENTR

9     9.942 B2     BEND

9     9.942        EXIT

9     9.942        DRIFT        .000            658.000           .000            .000            .000         659.000     	39.66101

9     9.942        ROTAT

9    11.178        DRIFT        .000            357.000           .000            .000            .000         358.000     	39.30301

9    11.492 Q8     QUAD

9    11.492        DRIFT        .000            551.000           .000            .000            .000         551.000     	38.75201

9    12.109        DRIFT        .000           2291.000           .000            .000            .000        2291.000     	36.46101

9    12.423 Q9     QUAD

9    12.423        DRIFT        .000            668.000           .000            .000            .000         668.000     	35.79301

9    13.068        DRIFT        .000           6235.000           .000            .000            .000        6235.000    	 29.55801

9    14.068 POSI   SEPA

9    14.068        DRIFT        .000            557.000           .000            .000            .000         557.000    	 29.00101

9    14.713        DRIFT        .000            385.000           .000            .000            .000         387.000     	 28.61401

9    15.713 KICK   SEPA

9    15.713        DRIFT        .000           1023.000           .000            .000            .000        1023.000     	 27.59101

9    16.299 ANSP   DRIFT        .000            108.000           .000            .000            .000         108.000     	27.48301

9    16.613 Q10    QUAD

9    16.613        DRIFT        .000             17.000           .000            .000            .000          18.000     	27.46501

9    16.809        DRIFT        .000            181.000           .000            .000            .000         182.000     	27.28301

9    17.123 Q11    QUAD

9    17.123        DRIFT

9    17.319        DRIFT

9    17.633 Q12    QUAD

9    17.633        DRIFT

9    18.433 SLXY   DRIFT

9    19.633        DRIFT

9    19.999 Q13    QUAD

9    19.999        DRIFT        .000            699.000           .000            .000            .000         699.000     	26.58401

9    20.131        DRIFT        .000           2494.000           .000            .000            .000        2494.000     	24.09001

9    20.497 Q14    QUAD

9    20.497        DRIFT        .000             52.000           .000            .000            .000          53.000     	24.03701

9    20.629        DRIFT

9    20.995 Q15    QUAD

9    20.995 AQ15   DRIFT

9    21.995 F      DRIFT

# Peter’s tunes and variants

## jt1.dat: simple improved ? jt tune

Figure 14: jt1 and jt

Main difference:

jt:    5.00           'Q11 '      0.31400     0.82379    12.90000;

jt1:    5.00           'Q11 '      0.31400     0.80500    12.90000;

gives 50% better SLXY x focus.

However, x resolution worse in revmoc!

## pnb.dat: new nb kick tune

Figure 15: pnb1.dat after 1st  order focusing

Figure 16: pnb.dat optimized in 2nd order (by hand)

Figure 17: phase space for pnb

/*PLOT* pnb/

0

1.000000       '    '      0.30000   150.00000     2.00000    30.00000

0.00000     4.00000     0.02800;

17.             '    ' ;

3.0            '    '      0.59750;

5.00           'Q1  '      0.47400     0.59112    12.00000;

3.0            '    '      0.11950;

5.00           'Q2  '      0.47400    -0.41200    12.00000;

3.0            '    '      0.62000;

5.00           'Q3  '      0.47400     0.18584    12.00000;

3.0            '    '      0.72130;

16.00           '    '      5.00000     9.00000;

16.00           '    '      7.00000     0.30000;

16.00           '    '      8.00000     2.80000;

2.0            '    '     24.00000;

4.000          'B1  '      0.65950     1.48302     0.00000;

2.0            '    '     24.00000;

3.0            '    '      0.40130;

3.0            'SEX1'      0.32000;

3.0            '    '      0.41350;

3.0            '    '      0.13500;

5.00           'Q4  '      0.39000     0.63212    12.50000;

3.0            '    '      0.21000;

5.00           'Q5  '      0.39000    -0.45737    12.50000;

3.0            '    '      0.14150;

3.0            '    '      0.14150;

5.00           'Q6  '      0.39000    -0.27195    12.50000;

3.0            '    '      0.26000;

5.00           'Q7  '      0.39000     0.52178    12.50000;

3.0            '    '      0.82850;

3.0            'SEX2'      0.32000;

3.0            '    '      0.37630;

20.0            '    '    180.00000;

2.0            '    '     22.65000;

4.000          'B2  '      0.69420     1.40891     0.00000;

2.0            '    '     12.85400;

20.0            '    '    180.00000;

3.0            '    '      1.23620;

5.00           'Q8  '      0.31400     0.30   12.90000;

3.0            '    '      0.61700;

5.00           'Q9  '      0.31400     -0.185   12.90000;

3.0            '    '      0.64500;

3.0            'SEPA'      1.00000;

3.0            '    '      0.70000;

3.0            'KICK'      1.00000;

3.0            '    '      0.58600;

5.0B           'Q10 '      0.31400    -0.42    12.90000;

3.0            '    '      0.19600;

5.01           'Q11 '      0.31400     0.77    12.90000;

3.0            '    '      0.19600;

5.0B           'Q12 '      0.31400    -0.42    12.90000;

3.0            'SLXY'      0.80000;

-10. -1.0 2.0 0.0 .001 /A12 / ;

-10. -3.0 4.0 0.0 .001 /A34 / ;

3.0            '    '      1.20000;

5.0A           'Q13 '      0.36600     0.61983    12.70000;

3.0            '    '      0.13200;

5.01           'Q14 '      0.36600    -.90  12.70000;

3.0            '    '      0.13200;

5.0A           'Q15 '      0.36600    0.61983   12.70000;

3.0            'F   '      1.00000;

-10. -1.0 2.0 0.0 .001 /A12 / ;

-10. -3.0 4.0 0.0 .001 /A34 / ;

SENTINEL

SENTINEL

Figure 18: unkicked

Figure 19: unkicked correlations

Figure 20: kicked with 1.5 kV/cm

Conclusion: Non-bend kick not impossible, but with present effort not convincing. Similar transmission as jt.

## pd.dat: increased transmission (?) jd tune

Figure 21: pd first and second order

Figure 22: pd: slit and target

Yet to be tested with revmoc. Final focus not ideal yet.

/*PLOT* pd/

0

1.000000       '    '      0.30000   150.00000     2.00000    30.00000

0.00000     4.00000     0.02800;

17.             '    ' ;

3.0            '    '      0.59750;

5.00           'Q1  '      0.47400     0.59112    12.00000;

3.0            '    '      0.11950;

5.00           'Q2  '      0.47400    -0.41200    12.00000;

3.0            '    '      0.62000;

5.00           'Q3  '      0.47400     0.18584    12.00000;

3.0            '    '      0.72130;

16.00           '    '      5.00000     9.00000;

16.00           '    '      7.00000     0.30000;

16.00           '    '      8.00000     2.80000;

2.0            '    '     24.00000;

4.000          'B1  '      0.65950     1.48302     0.00000;

2.0            '    '     24.00000;

3.0            '    '      0.40130;

3.0            'SEX1'      0.32000;

3.0            '    '      0.41350;

3.0            '    '      0.13500;

5.00           'Q4  '      0.39000     0.63212    12.50000;

3.0            '    '      0.21000;

5.00           'Q5  '      0.39000    -0.45737    12.50000;

3.0            '    '      0.14150;

3.0            '    '      0.14150;

5.00           'Q6  '      0.39000    -0.27195    12.50000;

3.0            '    '      0.26000;

5.00           'Q7  '      0.39000     0.52178    12.50000;

3.0            '    '      0.82850;

3.0            'SEX2'      0.32000;

3.0            '    '      0.37630;

20.0            '    '    180.00000;

2.0            '    '     22.65000;

4.000          'B2  '      0.69420     1.40891     0.00000;

2.0            '    '     12.85400;

20.0            '    '    180.00000;

3.0            '    '      1.23620;

5.00           'Q8  '      0.31400    -0.14759    12.90000;

3.0            '    '      0.61700;

5.00           'Q9  '      0.31400     0.17785    12.90000;

3.0            '    '      0.64500;

3.0            'SEPA'      1.00000;

3.0            '    '      0.70000;

3.0            'KICK'      1.00000;

3.0            '    '      0.58600;

5.00          'Q10 '      0.31400     0.45    12.90000;

3.0            '    '      0.19600;

5.00          'Q11 '      0.31400    -0.490        12.90000;

3.0            'SLXY'      0.70;

-10. -1.0 2.0 0.0 .001 /A12 / ;

-10. -3.0 4.0 0.0 .001 /A34 / ;

3.0            '    '      1.30000;

5.01          'Q12 '      0.36600      0.65    12.70000;

3.0            '    '      0.13200;

5.01          'Q13 '      0.36600     -0.60    12.70000;

3.0            'F   '      1.00000;

10. -1.0 2.0 0.0 .001 /A12 / ;

-10. -3.0 4.0 0.0 .001 /A34 / ;

-10. 12. 1. 0.0 .001 /r21 / ;

-10. 13. 4.0 0.0 .001 /r34 / ;

SENTINEL

SENTINEL

# Conclusion and tuning strategy

- Conclusions

  - 2nd order (chromatic aberrations) critically important

  - intensity tight

  - kicker possible

- Further work

  - How do we share the work

  - Study all transport tunes with revmoc/turtle

  - Stability of results ?

    - Sensitivity to changes

    - include curved poles and PSI fringe field

    - Full tracking calc (Zgoubi)

  - how to diagnose what’s going on during the test run

  - choice of momentum bin

    - measure energy distribution with calibrated Si det or mu+ edge (equivalent?)

    - higher momentum tune naturally reduces momentum bin

- Tuning strategy

  - beam hardware

    - establish channel hardware set-up

    - Jaap suggests use of sepa for kicking

    - Kicker geometry must be realistic, H and W

    - Triplet or doublet

    - vertical/horizontal kick

  - establish tuning/measuring plan