PC1  wire chamber.

The PC1 chamber will be used for  the tracking of incoming muons.

The PC1 contains 6 wire planes: 2 anodes and 4 cathodes.

The signals are read out in  both coordinates X-Y by making use of

two anode wire planes.

The geometry parameters of the PC1 are as follows:

HV cathode 1---55mu Fe wires spacing 1 mm

Anode 1--- 25mu W wires spacing 2 mm

HV cathode 2--- 55mu Fe wires spacing 1 mm

HV cathode 3--- 55mu Fe wires spacing  1 mm

Anode  2--- 25mu W wires  spacing  2 mm

HV cathode 4--- 55mu Fe wires spacing  1 mm

Anode-cathode gaps---4mm

Sensitive region in X directon ---50mm

Sensitive region in Y direction ---50mm

Total size PC1 in X direction ---140mm

Total size PC1in Y direction ---140mm

PC1 input window ---50mu mylar

PC1 exit window ---50mu mylar

PC1 gas composition: 50% Argon, 50% C2H6, 0.2% Freon

Gas thickness (35 mm)

The PC1 showed stable operation up to HV= -2.8 kV.

Figure shows the dependence of the efficiency versus

HV. This dependence has the wide plateau from 2.3 up to 2.7 kV.

The nominal high voltage was chosen to be 2.5 kV.

Figure.   PC1 efficiency versus  HV. The relative measurement of the efficiency

was carried out with muon  beam at momentum 37.7 MeV/c .