# Conductive Shield for TPC HV Cathode

Peter Kammel

# Problem and Method

This note investigates the geometry of a conductor frame to reduce ion accumulation on the insulating glass of the TPC HV cathode. The problem has been described in the MuCap Webpage, Garfield studies (Patel and Klein).

We simulate a conductive frame on top of the glass cathode with =1mm wires. Two geometries were studied with Garfield. The frame only (“HV shield”) and the frame with an additional potential wire at the vertical position of the HV cathode (“HV shield with additional wire”).

The results are plotted in tpcs.ps and atpcs.ps, respectively.

The figures are

| 1 | potential contours full tpc |
| --- | --- |
| 2 | quadrant right top |
| 3 | quadrant left top |
| 4 | quadrant left bottom |
| 5 | quadrant right bottom |
| 6 | zoom left top |
| 7 | zoom right top |
| 8 | TPC entrance vertical field distribution |
| 9 | hor. field distribution at shield height |
| 10 | “positron=ion” drift lines for track left top, hor |
| 11 | “positron=ion” drift lines for track left top, vert |
| 12 | “positron=ion” drift lines for track right top, hor |
| 13 | “positron=ion” drift lines for track right top, ver |

# Results and conclusions

- The additional frame protects well against ions from the region above the HV cathode (c.f. fig.10,12)

- Ion drift from the region left/right of the cathode is not completely eliminated. The additional potential, strung around the ceramic post, reduces the effect somewhat, but not dramatically 
(c.f. fig. 11,12).

- The outer edge of the metal frame sees the largest field (~55kV/cm) of the whole HV cathode, it should be carefully rounded off (fig.9)

We will see empirically whether the remaining effect is serious. If that is the case, on could either wrap the shield around the glass frame or use a metal plate for the HV cathode (I forgot why we did not do that in the first place).